Citizen Hyper AquaLand  PalmPilot  AquaLand Graph

Purpose

The Citizen Hyper AquaLand is severely limited in the amount of dive profile information it can store between downloads.  This isn’t much of a problem when diving locally but when on a dive trip you are stuck with either loosing the profile data or dragging along a PC (we’ve done both and neither is an acceptable solution).

If you are also an owner of a PalmPilot then you are in luck. Gary Lorenson GLorensen@aol.com has written a PalmPilot application which enables you to download you’re dives to a PalmPilot while on vacation then upon returning, you can download from the PalmPilot to Citizen’s AquaGraph program (just like it was coming from the watch but without the limitations).

Setup

You will need to build a null-modem cable to connect the PalmPilot’s cable to the AquaLand’s cradle (or you can buy a PalmPilot Modem cable and a 9-pin to 25-pin adapter).

To make a cable, you need some 5-conductor wire, a male DB-9 connector, and a male DB-25 connector.  Connections are as follows:

PalmPilot	AquaLand Cradle

(9-pin male)	(25-pin male)

---------	--------------------------------

2 (RxD) ------------  2(TxD)

3 (TxD) ------------  3(RxD)

4 (DTR) -----------  6(DSR)

5 (Gnd) ------------  7(SG)

6 (DSR) ----------- 20(DTR)

7 (RTS) ------------ 5(CTS)

8 (CTS) ------------ 4(RTS)

Load the DiveComputer application on the PalmPilot

- Obtain a copy of  “DiveComputer v1.3 for the PalmPilot”. This is available at http://members.aol.com/GLorensen

- Save divecomp.zip in it’s own directory and unzip it.

- Run the “PalmPilot Install Tool” click on “Browse” then point to DiveComp.prc (in the directory you unzipped to). Click on the file DiveComp.prc, “Open”, “Install”, and then “Exit”.

- HotSync you’re PalmPilot and the DiveComputer application will be loaded.

Saving Dives to the PalmPilot (while on an extended trip)

- Connect the PalmPilot to the AquaLand cradle using the adapter you made

- Put the watch in transfer mode by holding the Mode button down for 4-seconds.

- Select the dive you want to download (using fwd and rev buttons on the watch)

- Turn on the PalmPilot and touch the “Applications” button.

- Touch the “DiveComputer” application.

- Touch the “Read Data” box (the dive will now download into the Palm Pilot)

- Touch the “Dives” box to see the dive profile and insure the transfer was successful.

- Touch “Menu”, “Save”, and then give it a unique filename (i.e. “Cozumel Dive 1”).

- Repeat the previous steps until all dives are downloaded.

Notes: The watch memory is limited so download frequently.

Each AquaLand dive is stored as a separate database so the right/left scroll

keys will not work.

Transferring Dives to AquaLand Graph (after you return)

- Make sure HotSync is not running (if the icon is showing in the system Tray on your PC then click on it and click on “exit” – Note: HotSync will be restarted next time you re-boot).

- Start AquaLand Graph

- Check “Settings” and insure the correct serial port is selected (you want the port your PalmPilot is connected to).

- Click on “Receive” then “Receive” (Don’t hit OK yet!)

- Place the PalmPilot in is cradle

- Turn on the PalmPilot and touch the “Applications” button.

- Touch the “DiveComputer” application.

- Touch “Menu”, “Load”, and then select the dive to download (i.e. “Cozumel Dive 1”)

- Touch “Done”

- Touch “Menu”

- Touch “Transmit” then immediately hit the “OK” button on the PC screen

- Repeat the previous steps until all dives are downloaded into AquaLand Graph